ineo 164

Product Guide

15.02.11

Table of contents

Product Basic	3

Product Configuration	4

System Specifications	6

Copier Specifications	7

Printer Specifications (GDI)	9

Scanner Specifications	10

Consumables	11

Article Numbers and Compatibility	12

Service Part	13

Appendix	14

# Product Basic

| MODEL DEVELOPMENT CODE | Pegasus L |
| --- | --- |
| PROPOSED MODEL NAME | ineo 164 |
| SUCCESSOR MODEL FOR | new |
| ESTIMATED SALES START | 06/10 |
| DATAQUEST SEGMENT | I |
| ALSO AVAILABLE AS | KM bizhub 164 |
| TYPE | Desktop/Stationary Platen |
| MAX. (PEAK) MONTHLY PAGE VOLUME | 12.0 k copies |
| MAX. AVERAGE/PERMANENT MONTHLY PAGE VOLUME | 2.8 k copies |
| MACHINE LIFE | 168 k copies or 5 years, which ever comes first Scanner durability 100 k pages |
| MCBF Body (Mean Copies Between Failure) | 23 k copies |
| MCBJ (Mean Copies Between Jam) | 2 k copies or more |
| MCBV (Mean Copies between Visit) | no data |
| MTTR (Mean Time To Repair) | Less than 30 min. |
| OPERATING CONDITIONS | 2 pages/job, 6 % coverage |
| SAFETY & ENVIRONMENT |  |
| English/German Data Sheets | Downloadable in the dBox |

Standard paper weight for systems is 80 g/m2 for monochrome machines and 90 g/m2 for full colour ones. Since using these standards

result in optimum system performance, all specifications on performance, reliability, maintenance and replacement based on those stan-

dard papers. Using thick papers, labels, transfer and polyester films continuously can increase the wear and tear on replacement parts

and consumables – meaning your servicing costs will go up.

Use the bypass or the lower tray from the paper feed unit when working with materials having such characteristics as polyester films,

textile transfer films and labels. Otherwise, you may be faced with double feeding or a higher number of feeding jams.

If possible use subtray when working with materials having such characteristics as polyester films, textile transfer films and labels. Roller-

marks on the copy or print may appear or increased jams can occur. Standard paper weights yield the best possible copy and print results.

Using higher grammage may influence the quality of the output. Using standard paper weights also guarantees that double sided copies

and prints are made reliably. Different characters of films as well as thick, thin and coated papers may increase the jam rate for double

sided copies and prints.

# Product Configuration

| SYSTEM DIMENSIONS |  |
| --- | --- |
| ineo 164 ineo 164 + MB-503 Unit: mm (Inch) |  |

| SYSTEM CONFIGURATION |  |
| --- | --- |
| Standard - 250 sheet Universal Cassette Tray - Original Cover Option - 100 sheet Multi Bypass Tray MB-503 - USB Device Server |  |

# System Specifications

| UNIVERSAL CASSETTE TRAY | Standard |
| --- | --- |
| Capacity | 250 sheets universal (80 g/m²) |
| Paper Format | A5 – A3 (CD: 90 to 297 mm/FD: 140 to 432 mm) |
| Paper Weight | Plain Paper: 64 – 90 g/m² Thick Paper: 91 – 157 g/m² |
| Max. Paper Capacity | 350 sheets (Universal Cassette Tray + Multi Bypass Tray) |
| Remark | OHP, postcards , envelopes and labels are not supported |
| MULTI BYPASS TRAY MB-503 | Option |
| Capacity | 100 sheets universal (80 g/m²) |
| Paper Format | A5 – A3 (CD: 90 to 297 mm/FD: 140 to 432 mm) |
| Paper Weight | Plain Paper: 64 – 90 g/m² Thick Paper: 91 – 157 g/m² |
| Remark | OHP, postcards , envelopes and labels are not supported |
| USB DEVICE SERVER | Option |
| Remark | For network printing/scanning |

# Copier Specifications

|  | ineo 164 |
| --- | --- |
| WARM-UP TIME Main Power ON | 29 sec. or less |
| Sub Power ON | No Sub Power Switch |
| FIRST COPY | 8 sec. or less (A4) |
| COPY SPEED |  |
| Plain Paper | 16 cpm (A4 crosswise) 9.1 cpm (A3) |
| Thick Paper | 7 cpm (A4 crosswise) 5.3 cpm (A3) |
| RESOLUTION Reading/Writing | 600 x 600 dpi |
| GRADATION | 256 |
| COPY RAM | 32 MB (standard & max.) |
| ZOOM | 50 % - 200 % adjustable in 1 % steps |
| MULTI COPY | 1 – 99 |
| EXPOSURE CONTROL | Manual: 9 steps, Text, Photo, Photo/Text, Auto: Text, Text/Photo |
| VOID AREA | 4 mm all edges |
| INNER OUTPUT TRAY CAPACITY | 250 sheets |
| POWER REQUIREMENT | 220 – 240 V : 3.5 A, 50 – 60 Hz |
| POWER CONSUMPTION | Max.:800 W Copying: 253 W Stand by: 70 W Sleep mode: 1.8 W |
| COPIER DIMENSIONS (W x D x H) mm | Standard: 570 x 531 x 449 Incl. MBT: 606 x 531 x 449 |
| COPIER WEIGHT (kg) | 23.5 (w/o Toner & Developer) |

| FEATURES |  |
| --- | --- |
| 2in1/4in1 | yes |
| Copy Function Memory | yes (2) |
| Power Save | yes (sleep mode & stand by) |
| Book Separation | yes |
| ID Copy | yes |
| Sort/Group | yes |
| Electronic Sort | yes |
| Criss/Cross Sort | yes with optional MB-503 |
| Paper Type Selection | Plain / Thick1 / Thick 2 / Recycled |

# Printer Specifications (GDI)

| TYPE / VERSION | GDI Controller |
| --- | --- |
| PRINT SPEED |  |
|  | 16 ppm A4 crosswise / 9.1 ppm A3 / 7 ppm A4 thick paper |
| PRINT RESOLUTION | 600 x 600dpi |
| MAX PRINTABLE AREA | 294 x 420 mm minus 4 mm on all edges |
| PRINT RAM | 32 MB (shared system memory) |
| HOST INTERFACE | USB 1.1, USB 2.0 |
| SUPPORTED OS |  |
| Server | Windows 2000 / 2003 Server / 2008 Server Windows 2003 Server x 64 Edition |
| Client | Windows 2000 / XP / Vista / Windows7 Windows XP / Vista / Windows7 x64 Edition |

# Scanner Specifications

| SCAN FUNCTIONS |  |
| --- | --- |
| Selection of Original Image Quality | Text / Photo |
| Scan Speed | Flatbed Scan only |
| Scan Size | Max. A3 |
| Interface | USB 1.1, USB 2.0 equivalent |
| Supported OS | Windows 2000 / XP / Vista / Windows7 / 2003 Server / 2008 Server |
| Driver | TWAIN Driver |
| Scan Function | TWAIN |
|  | Real Time Mode (Pull) |
| Application Software | TWAIN Driver only |
| SCAN UNIT |  |
| Scan Method | CIS (7488 pixels) |
| Light Source | LED |

# Consumables

| TONER for ineo 164 |  |
| --- | --- |
| Name | TN116 |
| Concept | Bottle |
| Weight | 500 g (281.5 g Toner) per bottle |
| Yield | 11.0 k copies / prints (6% coverage) per bottle |
| Type | HD polymerized Toner |
| Units per carton | 2 bottles |
| DEVELOPER for ineo 164 |  |
| Name | DV116 |
| Weight | 200 g |
| Yield | 55 k copies / prints |
| Units per carton | 1 bag |
| PC-DRUM for ineo 164 |  |
| Name | DR114 |
| Yield | 55 k copies / prints |
| Units per carton | 1 drum |

Important: The yield values listed in the above table indicate the average life time of the Developer, Drum and

Toner. The life time is very much depending on printing conditions (coverage, paper size, etc.), differences

in the printing method (continuous or one page printing) and type of media (plain paper, thick paper, films, etc.).

# Article Numbers and Compatibility

| MACHINE | Article Number |  |
| --- | --- | --- |
| ineo 164 (230V Version) | A0XX121 |  |
| ineo 164 (120V Version) | A0XX1P1 |  |
| ACCESSORIES | Article Number | Compatibility |
| Multi Bypass Tray MB-503 | A1XRWY1 | --- |
| USB Device Server | 9960ASX1000U | --- |
| CONSUMABLES |  |  |
| Toner TN116 | A1UC0D0 | --- |
| Developer DV116 | A1UC5D0 | --- |
| PC Drum DR114 | 4021029801 | ineo 161/210, ineo 163/213 |

# Service Part

Maintenance and Firmware Upgrade

Please refer to the related Service Manual.

Notebook recommendation for Service support (minimum configuration)

Hardware:

Notebook, 2GB RAM, 250GB HDD (at least 30 GBs free HDD space when fully configured),

Optical Drive (CD/DVD Read & Write), PCMCIA Slot, 10/100Mbit Ethernet, USB 2.0

Software:

WinXP (rather than Vista or Windosw 7), MS-Office, Acrobat Reader, Burning Software (Nero Suite,

IBM Record Now etc.), FTP Client (Filezilla etc.)

Periodical Replacement Parts

# Appendix

| Productivity Chart (Simplex 1:1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Universal Cassette Tray |  | Multi Bypass | Thick Paper Mode |
| A3 | 9.1 |  | 9.1 | 5.3 |
| B4 | 10.3 |  | 10.3 | 5.7 |
| A4 L | 12.2 |  | 12.2 | 6.3 |
| A4 C | 16.0 |  | 16.0 | 7.0 |
| A5 L | 16.0 |  | 16.0 | 7.0 |
| A5 C | 20.4 |  | 20.4 | 8.0 |
| First Copy Time |  |  |  |  |
| Size |  | Time (sec. or less) |  |  |
| A3 |  | 10.8 |  |  |
| B4 |  | 10.0 |  |  |
| A4 L |  | 9.3 |  |  |
| A4 C |  | 8.0 |  |  |
| A5 L |  | 8.0 |  |  |

| What is delivered with the machine? |  |  |
| --- | --- | --- |
| Parts | Packed in Japan | Added in Europe |
| Toner | no | --- |
| PC-Drum | yes | --- |
| Developer | yes | --- |
| Feeder | N/A | --- |
| Original Cover | yes (standard) | --- |
| Copy Tray | incorporated | --- |
| Printed LRI incl. DVD with Driver & User’s Guide | --- | yes |
| Face Sheets | N/A | N/A |

www.develop.eu

Note!

Since changes in technical features are

the order of the day in the world of

digital copiers and printing technology,

please note that the technical data

provided in this material was issued in

April  2010. We reserve the right to

make changes to the technical design.

Specifications refer to maximum

performance under standard conditions.

No guarantee is provided in

relation to any of the data supplied.

Your Develop Marketing