# Readme.doc

Application note for using Realdiag.exe on EN55022:1998 conducted emission test (Realdiag.exe v1.7)

## Introduction

This program is used for generating traffic that meet the EN55022:1998 conducted emission test required.

Base on the EN55022:1998/CISPR 22:1997, in order to make reliable emission measurements representative of high LAN utilization, it is only necessary to create a condition of LAN utilization in excess of 10% and sustain that level for a minimum of 250ms. The content of the test traffic should consist of both periodic and pseudo-random messages in order to emulate realistic type of data transmission.

Realdiag.exe is a program that design for the test condition suitable for the test specified above and use only on Realtek series of Ethernet/Fast Ethernet products such as NICs, PHYs, etc.

## Environment

This program can be used on Windows 98 system.

## Install

You will  need 2 stations for the test, one for hosting the DUT and the other ack as test partner to response for the traffic.

To install the test programs, simply copy all 3 files ( Realdiag.exe, Realpkt.vxd, Realpkt5.vxd ) to a new directory on both DUT and the test partner.

## To use the program on test

First execute the realdiag.exe at both 2 stations, and will show as below, choose the adapter under test then press  “Open” button.

Then it will shows an environment screen as below,

Select the test pattern as Pseudo-Random or Periodic, adjust the Tx pattern length to meet the 10% traffic level ( about 300 or 400 byte), then press “Start” button at both side. Because this program is based on the NDIS driver information and it is not a ping-pong test scheme, Rx packet loss may occur. The proper measurement of traffics is simply recording the transmission traffic at both the DUT and the test partner, and the summation of the  traffics on these 2 stations must be larger than 10%.